Pictorial guide to Re-setting the tracking run.
Switch the drive motor off.
[image: image1.png]


  
Lift the platform table edge    
 [image: image2.png]


      
Tilt carefully anti-clockwise        
 [image: image3.jpg]



Re-set completed

Re-setting the platform

1) Switch the motor off and now re-set the tracking platform to the start position by lifting the drive side North edge of the platform table (CAREFULLY AND SLOWLY ) about 5-10mm ( black arrow) and tilting the platform table+telescope  assembly ANTI-CLOCKWISE( red arrow)  as viewed from the front and ensure the drive roller is not contacting the drive sector during manual table movement , until it hits the “start run”  position stop. Once you have lifted the edge, you will not need much force to tilt the platform table+ telescope and it will have significant inertia due to its mass, and a significant arc is involved so be careful. Taking about 5 seconds to perform this tilt slowly is about right.

2) You can now locate your next target, and then switch the motor drive on again to recommence tracking.

