GSO Newtonian - Aperture 150mm - Focal Length 750mm
	A multi-purpose advanced telescope for beginners and advanced observers alike. Excellent image quality.


	GSO Newtonian 150mm aperture on Sky View equatorial mount - an excellent entry level telescope


	
	GSO Newtonian 150mm Aperture, 750mm Focal Length

This GSO Newtonian is an excellent multi-purpose telescope.

It combines a very good mechanical quality with an exceptional optical quality. Moon and the planets are depicted full of contrast and detail. You will be able to see Saturn's Cassini division and details in Jupiter's cloud belts.

150mm aperture offer a lot of light for serious deep sky observing. Countless gas nebulae, star clusters and galaxies are within reach of this telescope. The Great Andromeda Galaxy will be seen as an extended cloud with its satellite galaxies.

In our opinion the optics of GSO Newtonians are better than those of Chinese make. The parabolical mirrors are very precisely figured and have a smooth surface for optimum contrast.


	Astro Imaging With The GSO Newtonian 150/750mm

For photographing gas nebulae and galaxies we recommend the T2 adaptor for the GSO Newtonian: The "GS-Fokaladapter". Just remove the 1.25" eyepiece holder and the small ring that connects it to the focuser and attach the T2 adaptor. Then all you will need is a normal T-ring for your SLR camera.

For the moon and the planets you need higher magnifications. This is achieved by eyepiece projection. The TS PAT adaptor is attached to the GS Fokaladapter. You can then insert many standard 1.25" eyepieces (orthoscopic eyepieces for best results) and once again attach your SLR camera with a standard T-ring.

We also offer adaptations for most digital cameras. Please request the adaptation for your specific camera.
	

Image:
TS Eyepiece Projection Adaptor TS PAT with optional T-ring


